# VSK-SW-Office-01 — Switch Configuration

## Overview

This configuration formalises VSK-SW-Office-01 as the primary access/distribution switch in the office. It preserves your current VLAN access/trunk setup and adds production-grade hardening: secure management (SSH), consistent edge templates (PortFast, BPDU Guard, storm control, port-security), DHCP Snooping, Dynamic ARP Inspection, and a management SVI on VLAN50 (Admin). Replace placeholder IPs/keys to suit your environment.

• Gi0/1 is the 802.1Q trunk upstream to the router/core (allowed VLANs 10,20,30,40,50).

• VLAN50 is the Admin/Management VLAN and provides the switch SVI address.

• SSH/AAA local used for management

## Port Role Summary

| Interface | Mode | VLAN / Purpose |
| --- | --- | --- |
| Fa0/1 – Fa0/6 | Access | VLAN20 (Staff) |
| Fa0/7 – Fa0/12 | Access | VLAN10 (Student) |
| Fa0/13 – Fa0/14 | Access | VLAN50 (Admin) |
| Fa0/15 – Fa0/20 | Access | VLAN30 (Management) |
| Fa0/21 | Access | VLAN20 (Staff) |
| Fa0/22 | Access | VLAN50 (Admin) |
| Fa0/23 | Access | VLAN10 (Student) |
| Fa0/24 | Access | VLAN40 (Guest) |
| Gi0/1 | Trunk | Allowed 10,20,30,40,50 (Upstream) |
| Gi0/2 | Unused | Shutdown (spare) |

## Running Configuration

! ===== System =====
version 15.0
service password-encryption
no ip domain-lookup
hostname VSK-SW-Office-01
ip domain-name vitalskills.local

! ===== Secure Mgmt (SSH/AAA) =====
username netadmin privilege 15 secret <SECRET>
ip ssh version 2
crypto key generate rsa modulus 2048

line vty 0 4
 login local
 transport input ssh
 exec-timeout 15 0
line vty 5 15
 login local
 transport input ssh
 exec-timeout 15 0
line con 0
 logging synchronous
 exec-timeout 10 0

! ===== VLANs =====
vlan 10
 name Student
vlan 20
 name Staff
vlan 30
 name Management
vlan 40
 name Guest
vlan 50
 name Admin

! ===== Spanning Tree =====
spanning-tree mode pvst
spanning-tree extend system-id
spanning-tree portfast default
spanning-tree bpduguard default

! ===== Edge Protection Templates =====
! (Applied implicitly via 'portfast default' & 'bpduguard default'. Add per-port extras as needed.)

! ===== Access Ports =====
interface range FastEthernet0/1 - 6
 description Staff Endpoints
 switchport mode access
 switchport access vlan 20
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface range FastEthernet0/7 - 12
 description Student Endpoints
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface range FastEthernet0/13 - 14
 description Admin Endpoints
 switchport mode access
 switchport access vlan 50
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface range FastEthernet0/15 - 20
 description Management Endpoints
 switchport mode access
 switchport access vlan 30
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface FastEthernet0/21
 description Staff Endpoint
 switchport mode access
 switchport access vlan 20
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface FastEthernet0/22
 description Admin Endpoint
 switchport mode access
 switchport access vlan 50
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface FastEthernet0/23
 description Student Endpoint
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

interface FastEthernet0/24
 description Guest Endpoint
 switchport mode access
 switchport access vlan 40
 spanning-tree portfast
 storm-control broadcast level 1.00
 storm-control multicast level 1.00
 storm-control unicast level 1.00
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky

! ===== Uplinks =====
interface GigabitEthernet0/1
 description Upstream Trunk to Router/Core
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 10,20,30,40,50
 spanning-tree link-type point-to-point
 spanning-tree guard root
 ip dhcp snooping trust
 ip arp inspection trust

interface GigabitEthernet0/2
 description Spare / Disabled
 shutdown

! ===== Management SVI =====
interface Vlan50
 description Admin/Management SVI
 ip address 10.86.96.122 255.255.255.252
 no shutdown

! ===== Infrastructure Security =====
ip dhcp snooping
ip dhcp snooping vlan 10,20,30,40,50
no ip dhcp snooping information option

ip arp inspection vlan 10,20,30,40,50

errdisable recovery cause psecure-violation
errdisable recovery cause bpduguard
errdisable recovery interval 300

! ===== Logging/NTP (adjust as required) =====
! logging host 10.86.96.122
! ntp server 10.86.96.122 prefer
! clock timezone AEST +10 0

end

• Access ports: PortFast + BPDU Guard, storm control, and sticky port-security restrict lateral threats and loops.

• Trunk uplink: Root guard helps stabilise STP; trust DHCP Snooping/DAI only on known uplinks.

• DHCP Snooping/DAI: Prevent rogue DHCP/ARP spoofing across user VLANs.