## 3.1.2 Logical Network Segmentation (VLANs and IP Addressing)

Logical network segmentation is implemented using Virtual Local Area Networks (VLANs) to separate network traffic based on user roles and operational requirements. This approach enhances security, optimizes bandwidth usage, and simplifies network management by isolating broadcast domains. Each VLAN is assigned a unique subnet using Classless Inter-Domain Routing (CIDR) to ensure efficient utilization of IP address space. The segmentation supports scalability, allowing future expansion without significant reconfiguration.

The VLAN design accommodates key organisational groups, including Students, Staff, Management, Guests, and Administration. Each group is allocated a defined range of IP addresses to match its expected usage. By implementing this design, inter-VLAN routing can be controlled through Layer 3 switches or routers with Access Control Lists (ACLs), ensuring only authorised communication between network segments.

| VLAN ID | VLAN Name | Network | CIDR | Usable IP Range | Broadcast Address | Available IP Addresses |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Student | 10.86.96.0 | /26 | 10.86.96.1 – 10.86.96.62 | 10.86.96.63 | 62 |
| 20 | Staff | 10.86.96.64 | /27 | 10.86.96.65 – 10.86.96.94 | 10.86.96.95 | 30 |
| 30 | Management | 10.86.96.96 | /28 | 10.86.96.97 – 10.86.96.110 | 10.86.96.111 | 14 |
| 40 | Guest | 10.86.96.112 | /29 | 10.86.96.113 – 10.86.96.118 | 10.86.96.119 | 6 |
| 50 | Admin | 10.86.96.120 | /30 | 10.86.96.121 – 10.86.96.122 | 10.86.96.123 | 2 |